QUESTION 1:

- Regarding SDG&E’s  customer service, regulator and meter (SRM) replacement costs,

  - Please explain the increases in replacement costs of service pipe, regulator and meter, between

    - costs presented in the 2009 BCAP workpaper,  “Hernandez SDG&E-LRMC_EH_workpapers_10.02.08”, Table “LRMCC-nco.4”, and

    - costs presented in the 2013 TCAP workpaper, “SDGE 2013 TCAP LRMC Customer Costs”, Table “LRMCC-nco.4”, sheet “MSA NCOp2”

  - Please provide all data itemizing or otherwise supporting the 2009 and current replacement cost estimates.

RESPONSE 1:

As illustrated in the table on the following page, most meter cost increases were contained within 15-30%.  However, replacement service line costs increased much more significantly between the two BCAPs.  According to the business unit, the driving factor for the service line increase was an increase in the trenching costs required for replacing a service line.  See the attachment for 2013 costs.  The source for the 2009 costs could not be located for this response.

| Flow Range | M&R Cost |  |  | Service Cost |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Medium Pressure | 2009 | 2013 | % Change | 2009 | 2013 | % Change |
| 0-275 | $148 | $179 | 21% | $5,027 | $7,339 | 46% |
| 276 - 425 | $313 | $369 | 18% | $5,027 | $7,339 | 46% |
| 426-630 | $666 | $687 | 3% | $8,309 | $13,290 | 60% |
| 631 - 800 | $972 | $1,119 | 15% | $8,309 | $13,290 | 60% |
| 801 - 1,100 | $1,124 | $1,296 | 15% | $8,309 | $13,290 | 60% |
| 1,101 - 1,500 | $2,068 | $2,713 | 31% | $8,309 | $13,290 | 60% |
| 1,501 - 2,000 | $2,431 | $3,171 | 30% | $8,309 | $13,290 | 60% |
| 2,001 - 3,000 | $2,457 | $3,196 | 30% | $8,309 | $13,290 | 60% |
| 3,001 - 5,000 | $3,196 | $3,864 | 21% | $13,535 | $31,305 | 131% |
| 5,001 - 7,000 | $3,921 | $4,459 | 14% | $13,535 | $31,305 | 131% |
| High Pressure |  |  |  |  |  |  |
| 0 - 940 | $1,775 | $2,395 | 35% | $8,309 | $13,290 | 60% |
| 941 - 1,050 | $2,776 | $3,544 | 28% | $8,309 | $13,290 | 60% |
| 1,051 - 2,000 | $2,735 | $3,582 | 31% | $8,309 | $13,290 | 60% |
| 2,001 - 2,700 | $2,799 | $3,582 | 28% | $8,309 | $13,290 | 60% |
| 2,701 - 4,000 | $2,824 | $3,607 | 28% | $13,535 | $31,305 | 131% |
| 4,001 - 6,600 | $3,851 | $4,744 | 23% | $13,535 | $31,305 | 131% |
| 6,601 - 9,200 | $4,576 | $5,339 | 17% | $13,535 | $31,305 | 131% |
| 9,201 - 14,500 | $5,027 | $5,829 | 16% | $36,919 | $37,502 | 2% |
| 14,501 - 21,400 | $7,198 | $8,248 | 15% | $36,919 | $37,502 | 2% |
| 21,401 - 24,000 | $57,039 | $47,219 | -17% | $29,154 | $35,975 | 23% |
| 24,001 - 46,000 | $62,760 | $52,323 | -17% | $29,154 | $35,975 | 23% |
| 46,001 - 79,000 | $72,296 | $66,389 | -8% | $69,759 | $122,409 | 75% |
| 79,001 - 377,000 | $112,797 | $86,426 | -23% | $86,192 | $149,297 | 73% |
| 377,001 - 600,000 | $225,593 | $0 |  | $115,293 | $152,545 |  |
| 600,001 - 4,250,000 | $620,381 | $0 |  | $1,123,270 |  |  |
| > 4,250,000 | $1,353,558 | $0 |  | $1,045,886 |  |  |

QUESTION 2:

Regarding the relationship between SDG&E’s and SoCalGas’s customer service, regulator and meter replacement costs,

  - Please explain the differences in replacement costs of service line, regulator and meter, between

    - costs presented in the 2013 TCAP workpaper, “SDGE 2012 TCAP LRMC Customer Costs”, Table “LRMCC-nco.4”, sheet “MSA NCOp2” , and,

    - costs presented in the 2013 TCAP workpaper, “SCG 2013TCAP LRMC Customer Costs”, sheet “Cust 5”.

  - Please provide all data itemizing or otherwise supporting the replacement cost estimates for both utilities.

RESPONSE 2:

- There are two primary differences between the SDG&E worksheet tab “MSA NCOp2” and the SoCalGas worksheet tab “Cust 5”. One is that the SDGE calculations are being done by meter type and the SoCalGas calculations are being done by customer class. The second is that the SDG&E worksheet is calculating the present value of the revenue requirement whereas the SoCalGas worksheet shows the weighted average capital cost.

The file for SDGE, tab “MSA NCOp2”, calculates the present value of the revenue requirement related to the replacement of service lines, regulators, and meters by meter type. This is then used to estimate of the total dollar amount of replacement costs of each meter type (see column M “Total SRM Annual Cost replacement”).

The file for SoCalGas, tab “Cust 5” contains the weighted average capital cost (not the present value of the revenue requirement) of the service line, regulators and meters by customer class (not by meter type as shown for SDG&E).

- The SoCalGas workpapers for meters and service lines were provided in Response 3 to TURN DR 2.  The costs for SDG&E are provided in Response 1 of this request.

QUESTION 3:

Regarding SoCalGas’ and SDG&E’s meter, regulator and service cost for new customer connection,

  - Please explain the difference in the cost of SDG&E and SoCalGas investment per customer for service lines, meters and regulators between

    - Costs presented in the 2013 TCAP workpaper, “SDGE 2013TCAP LRMC Customer Costs”, sheet “MSA Rental”

    - Costs presented in the 2013 TCAP workpaper, “SCG 2013 TCAP LRMC Customer Costs-NCO Rental Use”, sheet “Cust 5”.

  - Please provide all data itemizing or otherwise supporting the cost estimates for both utilities.

RESPONSE 3:

- Similar to Response #2, there are two primary differences between the SDG&E worksheet tab “MSA Rental” and the SoCalGas worksheet tab “Cust 5”. One is that the SDGE calculations are being done by meter type and the SoCalGas calculations are being done by customer class. The second is that the SDG&E worksheet is calculating the annualized revenue requirement whereas the SoCalGas worksheet shows the weighted average capital cost.

- The SoCalGas workpapers for meters and service lines were provided in Response 3 to TURN DR 2.  The costs for SDG&E are provided in Response 1 of this request.