Workpaper SCGWP100202A

Revision 1

Southern California Gas Company

Customer Programs Department

LivingWise Energy Savings Kit

Revision History

| Revision No. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 | February 2, 2010 | Initial LivingWise Energy Efficiency Kit workpaper | Dave Munk |
| 1 | December 27, 2011 | Revised the kit savings to 24.11 therms, derived from the approved EES Kit workpaper instead of outdated DEER05 values. EUL has changed from 9 yr to 10 yr per DEER08. | Chan Paek |

Measure Summary Table

| Measure Name | Work Paper RunID: WPSCXXXXXXXX- | CZ | Target Sector | Measure Electric End Use Shape (Load Shape) | EUL or RUL E3 Name: Measure Type (To Look up Measure Life) | NTG E3 Name: Program Type (To Look Up Net-to-Gross Ratio) | Unit Definition | Program Type (NEW, ROB, RET) | Gross Measure Cost ($/unit) | Incremental Measure Cost ($/unit) | Gas Savings (Therms) | Gross Unit Annual Electricity Savings (kWh/unit) | User Entered kW Savings per unit (kW/unit) | % Eligible for TOU AC Adjustment | Total Number of Units | Gross Realization Rate (GRR) | Delivery Methods |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LivingWise Kit |  | System | Residential | NA | 10 | 0.80 | kit | RET | 7.80 | 7.80 | 24.11 | 0 | 0 | 0 | 70000 | 1.0 | through elementary school classes |

# Table of Contents

Revision History	ii

Measure Summary Table	iii

General Measure & Baseline Data	2

1.1	Measure & Delivery Description	2

1.2	DEER Differences Analysis	2

1.3	Code Analysis	2

1.4	Measure Effective Useful Life	2

1.5	Net-to-Gross Ratios for Different Program Strategies	2

1.6	Gross Realization Rate	2

1.7	Time-of-Use Adjustment Factor	2

Section 2.	Energy Savings & Demand Reduction Calculations	2

Section 3.	Load Shapes	3

Section 4.	Base Case & Measure Costs	3

4.1	Base Case Cost	3

4.2	Gross Measure Cost	3

4.3	Incremental Measure Cost	3

References	4

General Measure & Baseline Data

The LivingWise Energy Savings Kit contains energy savings information and one 1.5 GPM low flow showerhead and two faucet aerators suitable for residential use. Gross annual therm savings for each Kit does not include behavioral savings (i.e., conservation) triggered by the educational materials included in each Kit.

  - Measure & Delivery Description

The LivingWise Energy Savings Kit is delivered to elementary school children through a curriculum based program effort.  The energy savings information is packaged with two faucet aerators and a 1.5 GPM low flow showerhead.

  - DEER Differences Analysis

DEER2005 provides old savings estimates for aerator and low-flow showerhead measures.  DEER showerhead savings value to account for the use of a 1.5 GPM unit versus the DEER-modeled 2.0 GPM unit.  However, Kit energy savings are based upon Therm Savings Kit Workpaper for lavatory faucet aerators and low-flow showerheads.

  - Code Analysis

Any relevant code restrictions are built into the DEER-based savings estimates.

  - Measure Effective Useful Life

2008 DEER data list a 10 EUL for aerators and for showerheads.

  - Net-to-Gross Ratios for Different Program Strategies

The 0.8 Net-to-Gross value is taken from DEER, i.e., the Net-to-Gross for “Other Residential Programs.”

  - Gross Realization Rate

See Section 2 below.

  - Time-of-Use Adjustment Factor

Not Applicable since electric savings are not included.

- Energy Savings & Demand Reduction Calculations

  - Energy Savings

Therm Saver Kit WorkpaperError: Reference source not found “SCGWP100309A_Rev3”shows the savings estimate to be 24.11 therms for a low-flow showerhead and two aerators before applying the installation rate, as shown below in Table I.  SCGWP100309A was reviewed by CPUC and was revised to address all comments made by CPUC.  Kitchen aerator savings are not included in this LivingWise Kit calculation.

- Savings Estimate using SCGWP100309A Rev3

|  | Savings (therms) |
| --- | --- |
| Low Flow Showerhead | 10.89 |
| Two Faucet Aerators | 6.61 x 2 |
| Total | 24.11 |

- Load Shapes

Not applicable since no electric savings are claimed.

- Base Case & Measure Costs

  - Base Case Cost

Base costs are assumed to be zero since the absence of aerators and low-flow showerhead are assumed to be the base condition.

  - Gross Measure Cost

The $7.80 gross measure cost per Kit is based upon historic LivingWise Kit costs.  It includes supplier discounts available to LivingWise because of bulk purchasing.

  - Incremental Measure Cost

The incremental measure cost is equivalent to the $7.80 gross measure cost.

References

Therm Savings Kit Workpaper, SCGWP100309A_Rev3