Load Balance and Score Card

Review and Acceptance

| Information Submitted: |  |  | - Load Balance Workpaper, EEA Report No. B-REP-06-599-07D, August 2006 - Load Balance & Score Card Tool, Version 1.4x, August 2006 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Submitted by: |  |  | Energy and Environmental Analysis, Inc. |  |  |  |  |
| Date: |  |  | August 1, 2006 |  |  |  |  |
| Program Affected: |  |  |  |  |  |  |  |
|  | X | Express Efficiency |  |  | X | Energy Efficiency Grant Program (EEGP) |  |
|  | X | Process Equipment Replacement (PER) |  |  | X | Custom Process Improvement (CPI) |  |
|  | X | Efficient Equipment Replacement (EER) |  |  | X | Recognition Program |  |
|  | X | Business Energy Efficiency Program (BEEP) |  |  |  |  |  |
|  |  | Other (please describe) |  |  |  |  |  |

The following individuals have reviewed the information cited above, and accept this information for determining energy consumption and/or energy savings related to energy efficiency measures.

| Tom DeCarlo, PE |  |  |  |
| --- | --- | --- | --- |
| Commercial & Industrial Program Manager |  | Approval Date |  |
| Southern California Gas Company |  |  |  |
| Eric Kirchhoff, PE |  |  |  |
| Energy Efficiency Engineering Supervisor |  | Approval Date |  |
| Southern California Gas Company |  |  |  |
| Christopher Goff, CEM, DGCP |  |  |  |
| Senior Account Executive |  | Approval Date |  |
| Southern California Gas Company |  |  |  |
| Arvind Thekdi, Ph.D. |  |  |  |
| President |  | Approval Date |  |
| E3M, Inc. |  |  |  |

Revision History

| Rev No. | Date | Description | Author |
| --- | --- | --- | --- |
| --- | 23 Mar 06Revision D | Original release | EEA, R. Tidball |
| A | 7 Apr 06 | Workpaper modified to match Version 1.14 of the Load Balance Tool | EEA, R. Tidball |
| B | 21 Apr 06 | Workpaper modified to match Version 1.3 of the Load Balance Tool | EEA, R. Tidball |
| C | 18 Jul 06 | Workpaper modified to match Version 1.4 of the Load Balance Tool. Primary change was the addition of an integrated Score Card to both the Tool and the Workpaper | EEA, R. Tidball |
| D | 1 Aug 06 | Minor revision at the end of Section 4 (Score Card Methodology) based on feedback from Arvind Thekdi. | EEA, R. Tidball |

B-REP-06-599-07D

Revision D

Load Balance & Score Card Workpaper

August 2006

Prepared for:

Prepared by:

Energy and Environmental Analysis, Inc.

www.eea-inc.com

| Headquarters | West Coast Office |
| --- | --- |
| 1655 N. Fort Myer Drive, Suite 600 | 12011 NE First Street, Suite 210 |
| Arlington, Virginia 22209 | Bellevue, Washington 98005 |
| Tel: (703) 528-1900 | Tel: (425) 688-0141 |
| Fax: (703) 528-5106 | Fax: (425) 688-0180 |

Acknowledgment

Dr. Arvind Thekdi of E3M developed the scoring criteria discussed in this workpaper for industrial process heating equipment.

Disclaimer

The Gas Company has made reasonable efforts to ensure all information is correct.  However, neither The Gas Company's publication nor verbal representations thereof constitutes any statement, recommendation, endorsement, approval or guaranty (either express or implied) of any product or service.  Moreover, The Gas Company shall not be responsible for errors or omissions in this publication, for claims or damages relating to the use thereof, even if it has been advised of the possibility of such damages.

TABLE OF CONTENTS

Page

1.	Introduction	1

2.	Load Balance	2

3.	Load Balance Methodology	3

4.	Score Card Methodology	4

5.	Tool (Excel Spreadsheet) Operation	5

6.	Summary	14

LIST OF TABLES

Page

Table 1.	Hypothetical Customer – Gas Equipment List and Operating Profile	3

Table 2.	Scoring Criteria	5

LIST OF FIGURES

Page

Figure 1.	Approach for Identifying and Calculating Energy Savings	1

Figure 2.	Illustration of Multiple Gas Systems Connected to a Common Meter	2

Figure 3.	Input Screen for Gas Consumption Based on Historical Data	7

Figure 4.	Input Screen for Gas Consumption Based on User Estimate	7

Figure 5.	Gas Equipment Data Entry Worksheet and Summary Table	8

Figure 6.	Load Balance Section	9

Figure 7.	Customer Report	10

Figure 8.	Bar Chart	10

Figure 9.	Pie Chart	11

Figure 10.	Selection Table for Gas Equipment to be Scored	11

Figure 11.	General Operating Conditions	12

Figure 12.	Survey Questions	12

Figure 13.	Recommended Energy Efficiency Enhancements	13

Figure 14.	Identification of Possible Energy Efficiency Opportunities	13

# 1.	Introduction

This Workpaper is developed from the perspective of an Account Executive (AE) that is interested in evaluating energy efficiency opportunities at an industrial customer’s site.  In general, the AE needs to follow three general steps:

- Determine gas consumption for individual process heating systems

- Conduct qualitative assessment of energy efficiency opportunities (generally want to focus on the gas systems identified in Step #1 that consume the most gas)

- Complete a detailed quantitative analysis for those energy efficiency measures that offer the best opportunities (typically focus on best measures identified in Step #2)

Tools have been developed to support each of these three steps.  The intent for the tools is to simplify the calculation process, and help AEs more quickly and consistently estimate energy efficiency savings.  The tools that have been developed include:

- Load Balance Tool (addresses Step #1)

- Score Card Tool (addresses Step #2

- Equipment Specific Tools (several separate resources that address Step #3)

Figure 1 illustrates how the tools are intended to be used.  In concept, the idea is to first run a load balance for the entire plant.  After balancing the load, a specific gas system can be chosen for analysis with the Score Card Tool.  The results of the Score Card Tool will show possible energy enhancement opportunities for the specific gas system chosen.  Based on the results of the Score Card, quantitative energy savings for a specific energy efficiency measure can then be computed using an equipment specific resource.

Score Card Tool

Load Balance Tool

Thermal Oxidizer Tool

Furnaces & Ovens Tool

Heat Recovery Tool

Excess Air Tool

Qualitative rank of energy efficiency opportunities

Annual gas use before implementation of measure (therms/yr)

Annual gas use after implementation of measure (therms/yr)

Like to Like Tool

(equipment replacement)

Figure 1.	Approach for Identifying and Calculating Energy Savings

In practice, the Load Balance Tool and the Score Card Tool have been integrated into a single application.  This workpaper focuses on the integrated Load Balance & Score Card (LB&SC) Tool.  The Load Balance & Score Card (LB&SC) Tool automates the gas use scaling process, and provides a consistent and quick method for reconciling estimated and actual gas use data.  In addition to helping Account Executives and other staff balance gas loads, the LB&SC Tool includes a scoring methodology, which provides qualitative guidance on opportunities for enhanced energy efficiency.

It is important to recognize that the LB&SC Tool does not yield a quantitative estimate for energy savings that may be derived as a result of implementing specific energy efficiency measures.  Rather, the LB&SC Tool is a starting point, and the user is referred to equipment specific calculation tools, which are available as separate resources, for detailed energy efficiency calculations.

# 2.	Load Balance

To determine the impact of an energy efficiency measure, it is necessary to start with an accurate estimate of the gas consumption before the measure is implemented.  Determining the gas consumption prior to measure implementation is often complicated by the common practice of connecting multiple pieces of gas equipment downstream of a single gas meter.  An illustration of this situation is shown in Figure 2.  In this example, six natural gas systems are connected to a single gas meter at a customer’s location.  In general, billing data provides accurate information regarding the total volume of gas that passes through the meter.  However, the gas systems that are connected downstream of the meter are generally not sub-metered, and therefore direct measurements of gas use for individuals pieces of equipment are typically not readily available.

SCG Meter

Boiler #2

Furnace #2

Incinerator #2

Incinerator #1

Furnace #1

Gas Equipment

Known Parameters

Gas Consumption at Meter (e.g., therms/month or therms/yr from billing records)

Connected Load for Individual Gas Systems (e.g., rated full load capacity in MBtu/hr or MMBtu/hr from equipment nameplate or manufacturer’s literature)

Unknown Parameters

Gas Consumed for Individual Gas Systems

Boiler #1

Natural Gas

Figure 2.	Illustration of Multiple Gas Systems Connected to a Common Meter

A common approach for estimating the gas use for a single piece of equipment is to estimate the expected operating time and average firing rate.  The average firing rate is frequently determine by using an estimated load factor along with the full firing rate capacity, or connected load.  With these parameters, the gas use for a single gas system can be estimated.  To illustrate this approach, hypothetical usage and firing rate parameters for the gas equipment in Figure 2 are shown in Table 1.

Table 1.	Hypothetical Customer – Gas Equipment List and Operating Profile

| Equipment | Connected Load (gas input at full capacity, MMBtu/hr) | Load Factor | Operating Schedule |  |  | Annual Gas Use (therms/yr) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (hrs/day) | (days/wk) | (wks/yr) |  |
| Boiler #1 | 1.0 | 0.16 | 10 | 7 | 52 | 5,840 |
| Boiler #2 | 1.0 | 0.16 | 10 | 7 | 52 | 5,840 |
| Furnace #1 | 2.0 | 0.20 | 10 | 5 | 46 | 9,225 |
| Furnace #2 | 3.0 | 0.20 | 10 | 5 | 46 | 13,838 |
| Incinerator #1 | 4.0 | 0.20 | 12 | 7 | 46 | 30,997 |
| Incinerator #2 | 5.0 | 0.20 | 12 | 7 | 46 | 38,746 |
|  |  |  |  |  |  | 104,486 |

In Table 1, note that the firing rate is expressed in terms of the connected load, or nameplate equipment rating, and the load factor.  The nameplate rating is generally known for the installed gas equipment, and represents the maximum (full capacity) firing rate.  The load factor is a term that accounts for part load operation of modulating equipment or the operation of on/off equipment.  For modulating equipment, the load factor represents the average firing rate (load factor multiplied by connected load) for the gas equipment during the operating schedule.  If the gas equipment is an on/off system, rather than a modulating unit, the load factor represents the fraction of time that the equipment is on during the operating schedule.

The operating schedule is often expressed in terms of three parameters:  1) hours per day, 2) days per week, and 3) weeks per year.  The product of these three operating schedule parameters and the nameplate rating and the load factor yields the annual gas use.

For the hypothetical example in Table 1, the total gas use for all six pieces of gas equipment is slightly over 104,000 therms/yr.  If billing records for the gas meter match this total, then the estimates for the operating time and load factors are probably reasonable.  However, rarely do the estimated gas use values match the recorded gas meter data.  For example, if billing records show that gas use in recent years has been closer to 80,000 therms, it is generally necessary to scale back the estimated gas consumptions values for individual gas systems.  The Load Balance & Score Card (LB&SC) Tool automates the gas use scaling process, and provides a consistent and quick method for reconciling estimated and actual gas use data.

# 3.	Load Balance Methodology

The LB&SC Tool calculates a scaling factor (SF) that is required to reconcile the gas use for each individual piece of gas equipment, such that the sum of the gas consumption for all equipment matches the metered or expected annual gas consumption.  The scaling factor is calculated as follows:

SF=100 Qmi=1NRi LFi hi di wi	Eqn-1

where

Qm – Metered gas use (therms/yr)

N – Number of pieces of gas equipment that are connected to common gas meter

- R – Input rating for gas system (MBtu/hr).  This value is generally equivalent to the nameplate rating of the gas system, and represents the maximum firing rate.

LF – Load factor of each gas system

h – Daily operating time (hrs/day)

d – Weekly operating time (days/wk)

w – Annual operating time (wks/yr)

After the scaling factor is calculated, the load factor for each piece of equipment is scaled by the same factor such that the gas use matches the metered or expected gas consumption.  The adjusted load factor is calculated as follows:

LFadj = (SF) X (LF)	Eqn-2

The methodology for the Load Balance Tool allows users to exclude specific pieces of gas equipment from the adjusted load calculation.  For example, if a piece of equipment is known to operate a specific number of hours in a year, the load factor for this equipment can be omitted from the adjustment calculation shown in Equation 1.  In this situation, the load factors for all equipment, except any omitted systems, will be adjusted with a uniform scaling factor such that the sum of the gas consumption for all equipment – both adjusted and non-adjusted – matches the metered (or estimated) gas consumption.

# 4.	Score Card Methodology

Dr. Arvind Thekdi developed a methodology for assessing opportunities for energy efficiency improvements to industrial process heating equipment.  The methodology is based on a set of criteria that can be applied to a wide spectrum of industrial process heating equipment.  These criteria include four questions regarding the type of operation, and eight questions regarding the equipment condition or operating procedures (see Table 2).

Table 2.	Scoring Criteria

Type of Operation

1. Mode (e.g., batch or continuous)

2. Heating type (e.g., gas fired)

3. Process type (e.g., fluid heating)

4. Process temperature (e.g., 800-1,400 F)

Equipment Condition / Operating Procedures

1. Excess air control

2. Furnace pressure control

3. Combustion air preheat

4. Heat recovery

5. Furnace temperature control

6. Furnace seals

a. doors

b. shell

c. other

7. Wall insulation

a. hot spots

b. exterior surface temp > 200 F

c. refractory damage

8. Load variations

All of the criteria are structured with multiple choice or yes/no answers.  Based on the user responses to the questions, specific energy efficiency options are scored, and recommendations are identified for qualitative energy efficiency enhancements.  Given that the focus of the LB&SC Tool is gas equipment, the “Heating type” is set to gas-fired.  All of the other parameters – with the exception of “Mode” and “Process type” – have an effect on the score.

# 5.	Tool (Excel Spreadsheet) Operation

The Load Balance Tool and the Score Card Tool are integrated into a single Excel workbook.  This workbook contains the four load balance worksheets and three score card worksheets:

- Load balance worksheets (tabs)

- Gas Equipment

- Report

- Bar Chart

- Pie Chart

- Score card worksheets (tabs)

- Score Card

- Score Chart

- Score Guide

Each of the seven worksheets is summarized in the following sections

5.1	Gas Equipment

The Gas Equipment worksheet is intended for use by Gas Company personnel.  This tab contains all input fields required to determine an appropriate annual gas use estimate for a specific gas system being considered for an energy efficiency measure.  The Gas Equipment worksheet has four main sections:

- Account Information

- Gas Use

- Gas Equipment

- Load Balance

Account Information

In this section, the user is asked to provide the customer name and the billing account ID number.  This section also includes general guidance on using the LB&SC Tool and managing data.  For example, blue font is used to signify user input fields or data values that can be edited by the user.  In this section, there is a Save As button that can be used to save the LB&SC Tool using a new file name.  Note that the standard Excel File / Save As feature is disabled (to ensure proper file saving, the LB&SC Tool requires additional check functions not found in the standard Excel File / Save As feature).  The standard Excel File / Save feature can be used to save the LB&SC Tool using the same file name.

When creating new files, users are instructed to include the version number of the LB&SC Tool (e.g., Ver 1.4) in the file name.  By keeping the version number in the name, printed copies of the LB&SC Tool will show the version number in footers of printed pages.  For reference, the version number of the LB&SC Tool is shown in Excel / File Properties menu.  The version number in the Excel / File Properties menu should never be changed.

Gas Use

The protocol for determining annual gas use is as follows:

- 1st Choice – Use 36 consecutive months of recent actual gas consumption data

- 2nd Choice – Use 12 consecutive months of recent actual gas consumption data

- 3rd Choice – Provide an estimated annual gas consumption value

Figure 3 shows the input screen for either 36 or 12 months of gas consumption data (1st and 2nd choices in preceding list).  A total of 36 months should be used whenever possible.  However, if a full 36 months is not available, then a consecutive 12 month period can be used.  Based on the monthly values (input as therms), the LB&SC Tool calculates an annualized gas use (therms/yr).  Note that months entered as a zero (0) are included in the annualized average, and months that are left blank (null) are excluded from the calculation.

Figure 3.	Input Screen for Gas Consumption Based on Historical Data

Figure 4 shows the input screen used for supplying an estimated annual gas use value (3rd choice listed above).  If the user selects this option, the user is asked to provide an annual estimate (therms/yr), and then select a reason why historical data is not being used.  A drop down box is provided for selecting a reason.  Following the drop down box, the user is asked to provide further explanation describing the need to use an estimated value rather than historical gas consumption figures.   In the explanation box, the user should clearly show how the estimated value was derived, citing references as appropriate.

Figure 4.	Input Screen for Gas Consumption Based on User Estimate

Gas Equipment

After determining the total annual gas use, the next step is to develop a list of all gas equipment located at the site.  The user builds this list by using the Data Entry Worksheet illustrated in Figure 5.

Figure 5.	Gas Equipment Data Entry Worksheet and Summary Table

In the Data Entry Worksheet, the user begins by making selections from two drop down boxes – Equipment Type and Equipment Use.  The Load Balance Tool contains a library of over 300 pieces of gas equipment corresponding to the Equipment Type and Equipment Use selections available through the two drop down boxes.

In the Data Entry Worksheet, the user needs to manually enter the connected load (MBtu/hr) and the quantity of gas systems.  The connected load corresponds to the nameplate rating, or maximum firing rate, for the gas system.  Typically, the quantity of gas systems will be one.  However, if there are multiple gas systems that have identical capacities, operating schedules, and load factors, the systems can be entered on a single line in the Data Entry Worksheet.  After the Data Entry Worksheet is complete, the user clicks the Add Equipment button and the equipment is added to the Gas Equipment Table that immediately follows the Data Entry Worksheet.

In the event that incorrect data is entered or the Gas Equipment Table needs to be revised, the user can delete a single gas system or all of the gas systems in the Gas Equipment Table using the Delete Selected and Delete All Equipment buttons, respectively.

The Gas Equipment Table illustrated in Figure 5 is initially populated with default operating time and load factor values.  However, these default values can be replaced with user specified values.  In addition to the operating time (3 columns) and load factor parameters, the user has the ability to edit the quantity, connected load, and description fields in the Gas Equipment Table (all entries in blue font can be revised by the user).

As noted, the LB&SC Tool contains default load factor values.  The load factor defaults reside locally within the LB&SC Tool.  These default load factors should be checked periodically against current load factors, which should be available online at:

http://candi.pacent.com/aev/self_study/ACFH/Load_Factors-frame.htm.

Load Balance

The load balancing section of the LB&SC Tool is shown in Figure 6.  In this section, the user has the ability to reconcile the annual gas use as determined in Section 2 (Gas Use) with the annual gas consumption calculated by summing the gas consumption for each gas subsystem specified in Section 3 (Gas Equipment).

Figure 6.	Load Balance Section

As described in the methodology section, the LB&SC Tool operates by scaling the load factor for individual gas subsystems to match the annual gas use determined for the entire site.  Before balancing the gas load, the user is prompted to indicate if each gas subsystem should be included in the load adjustment calculation.  In general, all equipment should be included in the load balance calculation.  However, in some cases (for example backup boilers or engine driven power generators) the user may desire to use a fixed load factor that is not adjusted.  For these cases, the user has the ability to exclude equipment from the load factor adjustment routine (select “N” in the Auto Adjust column to exclude equipment from being automatically adjusted).  After making the appropriate yes/no selections in the Auto Adjust column, click the Auto Adjust button located below the Load Adjustment Table, and the load will be balanced.

The LB&SC Tool keeps track of the default operating times and the default load factors, and compares these values against the user input operating time parameters and the adjusted load factors.  Deviations in operating hours are displayed in green and deviations in load factors are displayed in orange (color coded arrows, as shown in Figure 6, indicate values being compared).  The criteria used for checking deviations are described in the text immediately above the Load Adjustment Table (if text is not visible, click the Show Instructions button).

5.2	Report

The Gas Equipment worksheet, described in the preceding section, is intended for use by Gas Company personnel, and contains detailed input parameters required for load balance calculations.  In contrast, the Report worksheet contains a summary of key information.  The Report worksheet is structured such that it can be handed out to customers, and kept on file as a record of load balance calculations.  This worksheet includes an approval section (see Figure 7) for the gas customer and SCG management to acknowledge acceptance of the results, should the results be intended for input to an energy efficiency rebate application.

Figure 7.	Customer Report

5.3	Bar Chart and Pie Chart

The Bar Chart and Pie Chart tabs contain graphical results that may be useful for discussions with gas customers.  These graphs can be included with the Report worksheet if desired.

Figure 8.	Bar Chart

Figure 9.	Pie Chart

5.4	Score Card

After balancing the load, the energy efficiency options for individual gas systems can be evaluated.  This evaluation begins with the Score Card worksheet.  On the Score Card worksheet, the first two sections provide reference data and instructions to guide the user (no user input required).  The initial user input is required in Section 3, where the user selects the gas system to be examined (see Figure 10).

Figure 10.	Selection Table for Gas Equipment to be Scored

After selecting the gas system, the user answers three questions concerning the general operating conditions of the gas system.  These three questions are shown in  (note that the heating type is set to gas fired for all cases).  After answering the three general questions, there is a longer list of eight questions (see Figure 12).  For all questions, there are drop down boxes indicating the response choices.

Figure 11.	General Operating Conditions

Figure 12.	Survey Questions

Based on the user responses to the questions, the LB&SC Tool assigns numeric scores and weighting factors to each response.  A total weighted score is generated, and compared to the maximum score possible.  For the LB&SC Tool, the maximum score represents a situation where all best practices are being followed to achieve energy efficiency.  Specific areas that fall short of the maximum score represent opportunities for energy efficiency enhancements.  These possible energy efficiency enhancements are listed and in the LB&SC Tool (see Figure 13).

Figure 13.	Recommended Energy Efficiency Enhancements

5.5	Score Chart and Score Guide

The Score Chart worksheet contains a graph of the actual score and the maximum score for the system being analyzed.  Energy efficiency options that have a score less than the maximum represent possible areas to apply energy efficiency measures to achieve gas savings.

Figure 14.	Identification of Possible Energy Efficiency Opportunities

The Score Guide worksheet contains a written description of the common energy efficiency measures that can be applied to industrial process heating equipment.  This guide can be used to gain further explanation of the energy efficiency measures identified on either the Score Card or Score Chart worksheets.

# 6.	Summary

In summary, the Load Balance & Score Card Tool provides the user with two key results:

- Methodology for developing gas use estimates for equipment connected to a common meter

- Qualitative guidance on energy efficiency improvements that may be applied to individual gas systems.

To run the load balance portion of the LB&SC Tool, the user needs metered gas use data and equipment nameplate ratings (or reliable estimates of these parameters).  The program has default values for equipment operating schedules and load factors, which can be reviewed and modified if desired.  After equipment details are entered, the LB&SC Tool computes the estimated gas use for individual gas subsystems such that the sum of the gas use by the individual gas subsystems matches the annual gas consumption for the entire site.  The output of the load balance portion of the LB&SC Tool is an annual gas use estimate for individual gas subsystems.

To complete the score card portion of the LB&SC Tool, the user needs a general understanding of the configuration and condition of the gas equipment being analyzed.  The user answers a series of multiple choice questions related to the gas system, and the responses to these questions are then used to identify possible energy efficiency enhancements that could be applied.

It is important to recognize that the Load Balance & Score Card Tool does not yield a quantitative estimate for energy savings that may be derived as a result of implementing specific energy efficiency measures.  Rather, the LB&SC Tool is a starting point, and the user is referred to equipment specific calculation tools, which are available as separate resources, for detailed energy efficiency calculations.

The operating schedule accounts for 364 days in a year (24 X 7 X 52).  In Table 1, the annual gas use values are adjusted to account for 365 days (8,760 hours) in a full year.

In the original methodology developed by Dr. Thekdi, there are four choices for heating type: 1) gas, 2) electric, 3) steam, and 4) other.

There is a fifth section titled “Results” at the end of the Gas Equipment worksheet.  This short section directs the user to other worksheets that contain summary tables and charts.